Ex-Ante Geometric Safety Evaluation of The Planned Sitinjau Lauik Flyover in Accordance with PDGJ 2021

Wiwit Juwita*, Didin Kusdian, Widiyo Subiantoro

Master’s Program in Civil Engineering, Universitas Sangga Buana, Bandung, Indonesia

e-mail: wiwitjuwita023@gmail.com

Abstract

The construction of the Sitinjau Lauik Flyover represents a strategic solution to address accident-prone segments in an area characterized by extreme topography. However, its safety performance must be validated at the planning stage. This study aims to evaluate the geometric safety of the proposed flyover in accordance with the latest Indonesian Highway Geometric Design Guideline (PDGJ 2021) using an ex-ante approach. In addition, the study seeks to identify critical segments with limited safety margins that may require additional safety treatments before operation. The evaluated parameters include horizontal alignment, vertical alignment, and stopping sight distance (SSD) at a design speed of 40 km/h. The quantitative analysis indicates that all geometric parameters comply with the minimum requirements of PDGJ 2021, with curve radii ranging from 97 m to 2800 m (R_min = 57 m), superelevation values between 2.10% and 5.20%, SSD between 43 m and 54 m, and vertical grades from −1.00% to 13.00%. Although the design satisfies the prescribed standards, critical safety margins were identified at segments with radii ≤110 m and grades ≥12%, where SSD values approach the minimum threshold. The novelty of this study lies in the early-stage risk identification during the design phase in mountainous terrain. It is concluded that ex-ante evaluation is essential for determining the need for additional safety measures at locations with limited safety margins to mitigate accident risks before operation.

Keywords: Geometric safety; ex-ante evaluation; PDGJ 2021; Sitinjau Lauik Flyover; safety margin

- Introduction

Road safety remains a major concern in the global transportation system, with a high number of traffic accidents and fatalities reported annually [1]. Infrastructure-related factors, particularly geometric road design, significantly contribute to crash risk by influencing driver behavior and vehicle dynamics [2][3]. Parameters such as horizontal curve radius, stopping sight distance (SSD), superelevation, and longitudinal grade directly affect vehicle stability and the driver’s ability to anticipate potential traffic conflicts [2][4].

The concept of design consistency emphasizes the importance of harmonization among geometric elements to maintain stable operating speeds and minimize abrupt maneuvers [5][6]. Design inconsistencies have been shown to correlate with increased crash risk, particularly at locations where horizontal and vertical alignments interact on two-lane roads [7][8]. Studies on driving performance along horizontal curves also indicate that the combination of small radii and suboptimal superelevation can reduce operational safety levels [9]. Furthermore, the relationship between geometric characteristics and crash frequency has been examined through various statistical crash prediction models [10], reinforcing the critical role of geometric design evaluation in road safety analysis.

Preventive approaches in road safety have evolved through the concept of road safety audit, which enables the identification of potential hazards at the planning stage [11][12]. Safety evaluation conducted during the design phase, commonly referred to as the ex-ante Approach, is considered more effective and cost-efficient than interventions implemented after infrastructure becomes operational (ex-post) [13][14]. This Approach focuses on risk prevention before actual crashes occur and has become an integral component of international road safety practices [3].

In Indonesia, geometric road design is guided by the 2021 Highway Geometric Design Guideline (PDGJ 2021) as the most recent national standard [15]. The guideline provides technical requirements for horizontal alignment, vertical alignment, sight distance, and other safety-related parameters. However, studies specifically assessing design compliance with PDGJ 2021 through an ex-ante approach in infrastructure projects characterized by extreme topography remain limited. Most previous research has primarily focused on crash analysis or on design consistency evaluation of existing roads [16][8][10], with limited integration of evaluations based on the latest national standard at the planning stage.

This limitation is particularly relevant in the context of the planned Sitinjau Lauik Flyover, which is intended to improve safety and traffic performance along a segment characterized by steep grades and sharp curves. Given the geometric complexity and potential risks associated with alignment transitions [17], a comprehensive evaluation of the design parameters is necessary before construction.

Accordingly, this study is confined to the evaluation of geometric parameters of the proposed Sitinjau Lauik Flyover based on the designated design speed, including curve radius, superelevation, stopping sight distance (SSD), longitudinal grade, and vertical curve parameters in accordance with PDGJ 2021 [15]. The assessment is conducted using an ex ante approach, without relying on actual crash data.

In general, numerous studies have been conducted on road safety evaluation; however, most have focused on ex-post analysis or post-crash assessments [18][19]. Several previous studies evaluating geometric design in mountainous terrain have typically relied on older design standards [20], which may not fully address the safety performance of modern vehicles operating under extreme grade conditions [21][22]. Therefore, although the construction of the Sitinjau Lauik Flyover is projected as a technical solution to accident-prone segments along the corridor, preventive safety evaluation (ex-ante) based on PDGJ 2021 remains limited.

This study addresses the identified research gap by verifying compliance with minimum design standards and analyzing the adequacy of safety margins at critical design locations. The novelty of this research lies in integrating an ex-ante approach into the geometric evaluation of a flyover structure in extreme topographic conditions, ensuring that safety considerations are systematically incorporated before construction.

Based on the aforementioned background, this study aims to: (1) assess the level of compliance of the proposed geometric parameters with PDGJ 2021; (2) identify potential safety risks arising from geometric inconsistencies based on design consistency principles [5][7]; and (3) demonstrate the effectiveness of the ex-ante Approach as a safety evaluation instrument at the planning stage [11][14].

- Materials and Methods

2.1 	Time and Location of the Study

This study, conducted in 2025, focuses on the geometric design plan for the Sitinjau Lauik Flyover in Padang City, West Sumatra Province, Indonesia. The project has an approximate length of 2.781 km, beginning at STA 0+000 (X = 667391.948; Y = 9895070.441) and ending at STA 2+829 (X = 668315.443; Y = 9894931.762).

The alignment is situated along a national road corridor characterized by undulating to steep topography, dominated by horizontal curves and significant longitudinal grades. Such conditions necessitate a thorough evaluation of geometric parameters, particularly curve radius, superelevation, stopping sight distance (SSD), and longitudinal grade. The assessment was conducted based on the design documents before construction; therefore, the analysis adopts a preventive ex-ante approach.

Figure 1. Location Map of the Proposed Sitinjau Lauik Flyover

Source: Final Report, Detailed Engineering Design (DED) of the Design–Build Project for the Construction of Panorama 1 Flyover (Sitinjau Lauik 1).

2.2 	Research Materials

The research materials consist of secondary data derived from the design documents, including:

- Horizontal alignment data (curve radius, curve length, and superelevation);

- Vertical alignment data (longitudinal grade, vertical curve length, and K-value);

- Design speed specifications;

- The 2021 Highway Geometric Design Guideline (PDGJ 2021) is the evaluation standard [15].

This study does not involve laboratory experimentation; instead, it employs a technical analysis based on the geometric design documentation.

2.3 	Evaluation Approach

This study employs an evaluative-comparative ex-ante approach, defined as a safety assessment conducted at the planning stage before infrastructure operation. This Approach aligns with the principles of international road safety audits [11][3], and has been demonstrated to be effective in identifying potential risks during the design phase [14][23].

Planning-stage evaluation is considered more effective than ex-post approaches, as it enables risk prevention before crashes occur [14][23]. Furthermore, risk-based infrastructure assessment has increasingly gained prominence in contemporary road safety studies [7].

2.4 	Geometric Evaluation Parameters

The evaluated parameters were selected based on empirical evidence indicating that horizontal and vertical alignment elements significantly influence operational road safety [7][8][9]. The analyzed parameters include design speed, stopping sight distance, curve radius, inner radius, curve length, lane width, outer shoulder width, curve direction, available width, lateral clearance, and curve widening.

Previous studies have demonstrated that inconsistencies between the curve radius and the actual operating speed are correlated with increased crash risk [7][24]. Moreover, the interaction between horizontal and vertical alignment affects vehicle stability and crash probability [8][24].

The relationship between geometric characteristics and crash frequency has been modeled using various statistical approaches [10], underscoring the importance of geometric design evaluation in road safety.

2.5 	Analysis Procedure

The secondary data, consisting of geometric design documents, was obtained from the relevant authority. These documents include horizontal and vertical alignment drawings, tables of geometric design parameters, and information on the designated design speed. Data validation was conducted through cross-checking consistency among design documents, verifying units and parameter values against PDGJ 2021 [15], and examining the conformity of stationing (STA) and coordinate data to ensure the absence of numerical inconsistencies before analysis.

The analysis is carried out through the following stages :

- Identification of Design Parameters

Inventorying all geometric parameters from the design documentation.

- Determination of Reference Standards

Establishing the minimum and maximum allowable values based on PDGJ 2021 [15].

- Comparative Analysis

Comparing the design values with the prescribed standards to determine compliance status.

- Risk Interpretation

Parameters that do not comply with or approach the prescribed limits were analyzed for their safety implications using design consistency principles [7][24] and design-based safety evaluation approaches [8][25].

This Approach does not rely on actual crash data but instead focuses on potential design-related risks (design-based safety assessment), as recommended for planning-stage safety evaluations  [14][23]. The overall research workflow is systematically illustrated in Figure 2.

Start

Problem Identification

Secondary Data Collection

Data Validation and Verification

Determination of Evaluation Parameters

Comparative Analysis with PDGJ 2021

Safety Margin Calculation

Identification of Critical Segments

Ex-Ante Evaluation Synthesis

Conclusion and Recommendations

Finish

Figure 2. Flowchart of the ex-ante geometric safety evaluation process

2.6 	Ex-Ante Evaluation Framework

The ex-ante evaluation framework applied in this study consists of three main stages:

- Technical Compliance Evaluation

A quantitative assessment of the conformity of geometric parameters with the prescribed standards [15].

- Design Consistency Evaluation

An analysis of the relationship between geometric parameters and potential variations in operating speed based on the design consistency concept [7][26].

- Identification of Potential Risk

Determination of segments with higher risk potential based on combinations of parameters approaching or deviating from prescribed limits, in accordance with a risk-based infrastructure safety management approach [7][23].

This framework enables a systematic assessment of geometric design before construction and can be replicated in other infrastructure projects characterized by complex topographical conditions.

- Results and Discussion

3.1 	Horizontal Alignment Evaluation

Computation of the SCS curve components at PI-2

- 	Determination of tangent distances (A-PI) and (PI-B)

Point A coordinates →	x 	= 	667866.499

y	= 	9895097.505

PI coordinates →   	x  	=	667199.829

y  	=   9894740.345

Point B coordinates  →  	x   	=   666896.631

y   =   9894640.485

dA-PI =XPI-XA2+YPI-YA2

=667199.829-667866.4992+9894740.345-9895097.5052=756.315 m

dPI-B=XB-XPI2+YB-YPI2

= 666896.631-667199.8292+9894640.485- 9894740.3452=319.219 m

- Calculation of Spiral–Circle–Spiral (SCS) Curve Components

Rc	= 500 m

∆ 	= 9.950°

LSmin	= w.n1.ed∆ = 7.1.2,10.7 = 21 m

A spiral transition length (Ls) of 30 m was applied in the calculation.

Xs 	= Ls1-Ls240 Rc2 = 301-30240 × 5002 = 30 m

Ys 	= Ls26 Rc = 3026 ×500 = 0.3 m

s	= 90π LsRc = 90π 30500 = 1.789 m

p 	= 10026 Rc-Rc1-coss = 10026 ×500-5001-cos1.789 = 0.075 m

k	= Ls Ls340 Rc2-Rcsins = 30 30340 25002-500×sin1.789 = 15.000 m

Ts	=  (Rc + p) tan ½ ∆ + k =  (500 +0.075) tan (½ x 9.950°) + 15.000 = 58.531 m

Es	=  (Rc + p) sec ½ ∆ - Rc =  (500 +0.075) sec (½ x 9.950°) – 500 = 1.966 m

Lc	=  (Δ - 2 θs)180 x π x Rc =  (9.950°) - 2 x 1.789)180 x π x 500 = 56.830  m

Ltot 	= Lc + 2 Ls = 56.830 + (2 x 30) = 116.830 m

- Determination of Key Curve Point Positions

Sta STP1	 = 0+319.011

Sta TS	= (Sta A+dA-PI–TsPI1)–TsPI2  = (0+319.011+756.315–468.864) – 8.531

= 547.931	= 0+547.931

Sta SC	= Sta TS + Ls = 0+547.931 + 30

= 577.931	= 0+577.931

Sta CS	= Sta SC + Lc = 0+577.931 + 56.830

= 634.761	= 0+634.761

Sta ST        	=  Sta CS + Ls = 0+634.761 + 30

= 664.761	= 0+664.761

The summarized results of the horizontal alignment design calculations are shown in Table 1.

The evaluation results indicate that the proposed Sitinjau Lauik Flyover consists of 10 points of intersection (PIs), with curve radii ranging from 97 m to 2800 m at a design speed of 40 km/h. According to PDGJ 2021, the minimum required radius for this design speed is Rmin=57m. The comparative analysis shows that 100% of the curves satisfy the minimum radius requirement. Approximately 20% of the curves (2 out of 10 PIs, with radii of 97 m and 110 m) are relatively close to the minimum threshold, with R/R_min ratios of 1.70 and 1.93, respectively, and no curve falls below the prescribed standard. The smallest radius is observed at PI-1 (97 m), followed by PI-4 (110 m), while the remaining radii—150 m, 235 m, 250 m, 400 m, 500 m, and 2800 m—provide larger geometric margins.

Although all curves comply with the standard requirements, segments with radii ≤110 m exhibit the smallest safety margins and may increase centrifugal forces and reliance on lateral tire friction. Under operational conditions, particularly on descending sections, this configuration may elevate the risk of loss of control for heavy vehicles.

In addition to curve radius, superelevation (e) values range from 2.10% to 5.20%, specifically 2.10%, 2.50%, 3.50%, 4.40%, 5.00%, and 5.20%. All values remain below the general maximum limit of 8% for a design speed of 40 km/h, thereby meeting the standard requirements. However, at curves with smaller radii—particularly 97 m and 110 m—the combination of near-minimum radius and non-maximum superelevation may reduce the stability margin compared to curves with larger radii. Overall, the horizontal geometric parameters meet the minimum requirements of PDGJ 2021, with primary attention given to the smaller-radius segments that exhibit the lowest radius-to-standard ratios.

3.2 	Stopping Sight Distance (SSD) Evaluation

Curve Widening at PI-1 Main Road, STA 0+475.322

- 	Data and Design Specifications:

Design speed (V_R) 	= 40 km/h
Lane width 	= 3.50 m
Outer shoulder width 	= 2.00 m
Curve radius 	= 97 m
Curve length 	= 312.55 m
Transition curve length 	= 50 m
Longitudinal grade 	= 3%

- Determination of Stopping Sight Distance (SSD)

For the normal direction (increasing STA) with an ascending grade of 3%, the adopted stopping sight distance (SSD) is 43 m.

For the opposite direction (decreasing STA direction) with a descending grade of 3%, the adopted SSD is 49 m.

- Determination of Required Lateral Clearance

Calculation of the inner radius for the normal direction:

R =  Curve radius +½ (lane width) + ½ (pavement marking width) = 97 + 1.75 + 0.06 = 98.81 m

Calculation of the inner radius for the opposite direction:

R = Curve radius - ½ (lane width) - ½ (pavement marking width) = 97 -1.75-0.06 = 95.19 m

Calculation of lateral clearance for the normal direction:

M = R {1 – cos 28,65 JpR}= 98.81 {1 – cos 28,65  x 4398.81} = 2.33 m

Calculation of lateral clearance for the opposite direction:

M = R {1 – cos 28,65 JpR}= 95.19 {1 – cos 28,65  x 4995.19} = 3.14 m

The required lateral clearance at STA 0+475.322 with a curve radius of 97 m is therefore 3.14 m measured from the roadway centerline. Given that the available width is 1.75 m, the required curve widening is 1.61 m, which is rounded up to 2.00 m for practical implementation. The summary results of the curve widening analysis are presented in Tables 2 and 3.

The stopping sight distance (SSD) was calculated from the reaction and braking distances at a design speed of 40 km/h. According to PDGJ 2021, the minimum required SSD for this speed is approximately 40–45 m. The evaluation results indicate that the SSD values along the Sitinjau Lauik Flyover alignment range from 43 m to 54 m, thereby generally satisfying the minimum requirement. The lowest value, 43 m, remains within the minimum safety threshold, while the highest value, 54 m, provides a greater visibility margin.

However, at several locations characterized by relatively small curve radii (97 m and 110 m) combined with significant longitudinal grade variations, the effective SSD approaches the minimum prescribed limit. This condition may reduce driver reaction time in emergencies, particularly on descending segments where operating speeds may exceed the design speed. Therefore, although all segments numerically comply with the SSD requirement, certain locations exhibit relatively narrow visual safety margins due to interactions between horizontal curvature and variations in vertical alignment.

3.3 Vertical Alignment Evaluation

- 	Main Road (sag vertical curve)

Design Data and Specifications

Design speed (VR )  		= 40 km/h

Maximum Grade 		= 4 %

Perception-reaction time (a)	= 2,5 seconds

Deceleration rate (a)		= 3.4 m/s2

Calculation of SSD for Ascending Grade

JPH=VD3.6t+VD22×3.62×9.81 a9.81±G =403.62.5+4022×3.62×9.81 3.49.81+0.04 = 44.076  ≈ 45 m

JPH calculation decreases

JPH=VD3.6t+VD22×3.62×9.81 a9.81±G =403.62.5+4022×3.62×9.81 3.49.81+0.08 = 42.551 ≈ 43 m

Calculation of the Design Grade

G1 	= 4.00%

G2 	= 8.00 %

A = |4.00 - 8.00| = 4.00 %

Calculation of Vertical Curve Length (L) Based on the Minimum Design Control Value (K) for Comfort Criteria

a	= 0,05g m/s

g	= 9,8 m/s2

K= VD21296a= 40212960.05x9.8 =  2.520

L  =  K x A = 2.520 x 4.00 = 10.08 m

Calculation of Vertical Curve Length (L) Based on the Design Control Value (K) for Stopping Sight Distance

For S < L

K= S22002h1-h2

h1  = (driver eye height for a passenger car)

h2   = 0.1 meter (height of a stationary object on the roadway)

K= 452200(21.2-0.1) = 9.65

L  =  K x A = 9.65 x 4.00 = 38.615 meter     Not compliant

For S > L

K= 2SA-2002h1- 2h2 2A2= 2x454.0-20021.2- 20.1 24.02 = 14.910

L  =  K x A = 14.910 x 4.0  = 59.641 meter  Does not meet the requirements

Calculation of vertical curve length (L) based on tabulated design control values (K) for stopping sight distance:

L  =  K x A =   9.0 x 4 = 36 m

Based on the comfort criterion: L = 10.08 m

Based on the tabulated K-value:= 36  m

Considering economic and design practicality, the adopted vertical curve length is: L = 100 m

Calculation of external distance:

Ev= A.L800= 4% x 100800 = 0.005 m

- 	Main Road (Crest Vertical Curve)

Design Data and Specifications (PVI-9, Main Road, STA 2+453.387)

Design speed (VR )  	= 40 Km/Jam

Maximum Grade 	= 4 %

Perception–reaction time (t)  	= 2,5 s

Deceleration rate (a)	= 3.4 m/s2

JPH calculations are on the rise

JPH=VD3.6t+VD22×3.62×9.81 a9.81±G=403.62.5+4022×3.62×9.81 3.49.81+0.08 = 42.551 ≈ 43 m

JPH calculation decreases

JPH=VD3.6t+VD22×3.62×9.81 a9.81±G=403.62.5+4022×3.62×9.81 3.49.81-0.045= 43.86≈ 44 m

Crest Vertical Curve Calculation

Determination of Design Grade

g1 	= 8.0 %

g2 	= -4.5 %

A = |8.0 – (4.5)| = 3.5 %

L  =  K x A =  4 x 3.5 =  14 m

Based on K-Value (Passing Sight Distance Criterion)

L  =  K x A =  23 x 3.5 = 80.5m

Based on Drainage Requirement

L < 51 A , L < 51 x 3.5 = 178.5 m

Based on Passenger Comfort Criterion

L > 0.6 V, L > 0.6 x 40 = 24 m

Adopted Vertical Curve Length

Based on the graphical criterion (S = L) :	L = 30 m

Based on K-value (Stopping Sight Distance):	L = 24m

Based on K-value (Passing Sight Distance):	L = 80.5 m

Based on drainage requirement:	L < 178.5 m

Based on passenger comfort:	sL > 24 m

Considering economic and practical design aspects, the adopted value is :L = 60 m

Ev= A.L800= 4% x 60800= 0.00263 m

The summary results of the vertical alignment design calculations are presented in Table 4.

The summary of the vertical alignment evaluation indicates that the Sitinjau Lauik Flyover alignment consists of 14 vertical curve points with a uniform design speed of 40 km/h. Elevation increases from 379.211 m at STA 0+000.000 to 553.188 m at STA 2+899.299, resulting in an overall elevation difference of approximately 174 m along the alignment. Longitudinal grades (g₁ and g₂) vary between −1.00% and 13.00%, with maximum grades of 13.00% at STA 2+708.079 and 12.00% at STA 2+774.152. The algebraic grade difference (A) ranges from 0.000 to 0.0900, while vertical curve lengths (L_v) vary between 36 m and 100 m, and external distances (E_v) range from 0.000 m to 0.005 m. The K-values range from 4.000 to 200.000, with the highest value recorded at STA 0+911.492 and the lowest at STA 0+425.000.

From a normative standpoint, most vertical curve configurations satisfy the requirements for a design speed of 40 km/h. However, the presence of grades up to 13% has significant technical implications for heavy vehicle performance. In a 13% downgrade, the component of the gravitational force acting parallel to the roadway surface increases proportionally with vehicle weight. For heavy vehicles such as fully loaded trucks, this condition necessitates continuous braking effort to maintain the intended operating speed. Prolonged brake application on extended downgrades may increase brake system temperature, reduce the friction coefficient of the brake linings, and potentially trigger brake fade. This reduction in braking effectiveness may lead to actual stopping distances exceeding the design SSD, particularly when combined with relatively small-radius horizontal curves.

Furthermore, the combination of 12–13% grades and horizontal curves along several segments increases the vehicle’s kinetic energy before entering the curve. From an operational perspective, heavy vehicle drivers tend to apply more intense braking before negotiating curves, thereby increasing the thermal load on the braking system. When the vertical curve length is relatively short (L_v = 36–50 m) and associated with low K-values (e.g., K = 4.000), the grade transition becomes sharper, resulting in greater dynamic loading on the vehicle. This condition may amplify the risk of loss of control, particularly when the horizontal geometric margin at the same location is also limited.

Therefore, although the vertical alignment parameters meet the minimum requirements of PDGJ 2021, the presence of extreme grades up to 13% within the Sitinjau Lauik topographic context indicates that safety evaluation should not rely solely on compliance with standards. Instead, it should consider the interaction among grade magnitude, downgrade length, dominant vehicle types, and horizontal curvature within the same segment.

Table 1. Summary of Horizontal Alignment Design Results for the Proposed Sitinjau Lauik Flyover

| Beginning of the Project |  | No. PI |  | PI-1 |  | PI-2 |  | PI-3 |  | PI-4 |  | PI-5 |  | PI-6 |  | PI-7 |  | PI-8 |  | PI-9 |  | PI-10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | 667391.9 | V (km/jam) |  | 40 |  | 40 |  | 40 |  | 40 |  | 40 |  | 40 |  | 40 |  | 40 |  | 40 |  | 40 |  |
| Y | 9895070 | Type |  | SCS |  | SCS |  | SCS |  | SCS |  | SCS |  | SCS |  | SCS |  | SCS |  | FC |  | FC |  |
| Sta | 0+000.00 | A |  | 155°05’04” |  | 95°07’01” |  | 58°11’10” |  | 108°43’25” |  | 39°01’49” |  | 14°51’38” |  | 31°26’11” |  | 17°40’50” |  | 0°37’20” |  | 9°33’49” |  |
|  |  | R (m) |  | 97 |  | 500 |  | 150 |  | 110 |  | 400 |  | 250 |  | 235 |  | 250 |  | 2800 |  | 400 |  |
|  |  | A1 | A2 | 69.64 | 69.64 | 122.4 | 122.4 | 86.60 | 86.603 | 74.162 | 74.162 | 118.32 | 118.32 | 93.54 | 93.54 | 96.95 | 96.95 | 100.0 | 100.0 | - | - | - | - |
|  |  | Ts1 (m) | Ts2 (m) | 468.8 | 468.8 | 58.53 | 58.53 | 108.8 | 108.8 | 179.7 | 179.7 | 159.3 | 159.3 | 50.12 | 50.12 | 86.21 | 86.21 | 58.91 | 58.91 | - | - | - | - |
|  |  | Lc (m) |  | 212.553 |  | 56.832 |  | 102.331 |  | 158.734 |  | 237.483 |  | 29.842 |  | 88.937 |  | 37.145 |  | 30.407 |  | 66.767 |  |
|  |  | Ls1 (m) | Ls2 (m) | 50.00 | 50.00 | 30.00 | 30.00 | 50.00 | 50.00 | 50.00 | 50.00 | 35.00 | 35.00 | 35.00 | 35.00 | 40.00 | 40.00 | 40.00 | 40.00 | - | - | (mengikuti eksisting) |  |
|  |  | LT (m) |  | 312.553 |  | 116.832 |  | 202.331 |  | 258.734 |  | 307.483 |  | 99.842 |  | 168.937 |  | 117.145 |  | - |  | - |  |
|  |  | e max (%) |  | 5.20 |  | 2.10 |  | 4.40 |  | 5.00 |  | 2.50 |  | 3.50 |  | 3.50 |  | 3.50 |  | En |  | (mengikuti eksisting) |  |
|  |  | PI | X | 667866.499 |  | 667199.829 |  | 666896.631 |  | 666861.985 |  | 667223.759 |  | 667677.505 |  | 667841.961 |  | 668123.370 |  | 668237.403 |  | 668298.885 |  |
|  |  |  | Y | 9895097.505 |  | 9894740.345 |  | 9894640.485 |  | 9894321.635 |  | 9894357.985 |  | 9894798.762 |  | 9894891.083 |  | 9894880.629 |  | 9894912.366 |  | 9894930.200 |  |
|  |  |  | Sta | 0+475.322 |  | 0+606.463 |  | 0+925.451 |  | 1+238.153 |  | 1+543.281 |  | 2+164.733 |  | 2+352.917 |  | 2+631.035 |  | 2+748.709 |  | 2+812.725 |  |
|  |  | TS/SS | X | 667398.396 |  | 667251.423 |  | 666999.997 |  | 666861.794 |  | 667065.089 |  | 667641.549 |  | 667766.785 |  | 668064.492 |  | - |  | - |  |
|  |  |  | Y | 9895070.809 |  | 9894767.985 |  | 9894674.529 |  | 9894496.313 |  | 9894343.715 |  | 9894763.834 |  | 9894848.881 |  | 9894882.816 |  | - |  | - |  |
|  |  |  | Sta | 0+006.459 |  | 0+547.931 |  | 0+816.623 |  | 1+058.447 |  | 1+383.971 |  | 2+114.605 |  | 2+266.706 |  | 2+572.116 |  | - |  | - |  |
|  |  | SC/TC/CC | X | 667448.228 |  | 667224.840 |  | 666953.506 |  | 666853.779 |  | 667099.896 |  | 667667.211 |  | 667802.195 |  | 668104.479 |  | 668222.755 |  | 668266.748 |  |
|  |  |  | Y | 9895069.368 |  | 9894754.084 |  | 9894656.298 |  | 9894447.076 |  | 9894347.357 |  | 9894787.623 |  | 9894867.458 |  | 9894882.397 |  | 9894908.287 |  | 9894920.873 |  |
|  |  |  | Sta | 0+056.459 |  | 0+577.931 |  | 0+866.623 |  | 1+108.447 |  | 1+418.971 |  | 2+149.605 |  | 2+306.706 |  | 2+612.116 |  | 2+733.505 |  | 2+779.264 |  |
|  |  | CS/CT/CC | X | 667494.972 |  | 667172.633 |  | 666885.476 |  | 666948.688 |  | 667312.574 |  | 667691.111 |  | 667888.126 |  | 668141.335 |  | 668252.005 |  | 668332.123 |  |
|  |  |  | Y | 9894903.314 |  | 9894731.703 |  | 9894582.517 |  | 9894337.035 |  | 9894444.973 |  | 9894805.463 |  | 9894888.233 |  | 9894886.736 |  | 9894916.602 |  | 9894934.052 |  |
|  |  |  | Sta | 0+269.012 |  | 0+634.763 |  | 0+968.954 |  | 1+267.182 |  | 1+656.454 |  | 2+179.447 |  | 2+395.642 |  | 2+649.262 |  | 2+763.913 |  | 2+846.031 |  |
|  |  | ST/SS | X | 667453.209 |  | 667144.236 |  | 666871.070 |  | 666998.568 |  | 667338.030 |  | 667721.216 |  | 667928.112 |  | 668180.132 |  | - |  | - |  |
|  |  |  | Y | 9894876.090 |  | 9894722.035 |  | 9894534.702 |  | 9894337.732 |  | 9894468.899 |  | 9894823.300 |  | 9894887.883 |  | 9894896.427 |  | - |  | - |  |
|  |  |  | Sta | 0+319.012 |  | 0+664.763 |  | 1+018.954 |  | 1+317.182 |  | 1+691.454 |  | 2+214.447 |  | 2+435.642 |  | 2+689.262 |  | - |  | - |  |
|  |  | Azimuth |  | 86°44'09" |  | 241°49'13" |  | 251°46'14" |  | 193°35'04" |  | 84°51'39" |  | 45°49'50" |  | 60°41'28" |  | 92°07'39" |  | 74°26'49" |  | 73°49'29" |  |

Source: Final Report, DED of the Panorama 1 (Sitinjau Lauik 1) Flyover Project.

Table 2. Summary of Curve Widening Results for the Proposed Sitinjau Lauik Flyover
(STA 0+475.322 to 1+541.328)

| GEOMETRIC PARAMETERS | NORMAL | OPPOSITE | NORMAL | OPPOSITE | NORMAL | OPPOSITE | NORMAL | OPPOSITE | NORMAL | OPPOSITE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | STA 0+475.322 |  | STA 0+606.378 |  | STA 0+919.870 |  | STA 1+205.221 |  | STA 1+541.328 |  |
| Design Speed (Vr) | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
| Stopping Sight Distance (JPH/SS) | 43 | 49 | 44 | 52 | 44 | 52 | 45 | 51 | 43 | 54 |
| Curve Radius (R) | 97 | 97 | 500 | 500 | 150 | 150 | 110 | 110 | 400 | 400 |
| Inner Radius (Ri) | 98,81 | 95,19 | 501,75 | 494,75 | 144,75 | 151,75 | 108,19 | 111,81 | 394,75 | 401,75 |
| Curve Length (Lc) | 312,55 | 312,55 | 116,83 | 116,83 | 202,33 | 202,33 | 258,73 | 258,73 | 307,48 | 307,48 |
| Lane width | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 |
| Outer Shoulder Width | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Curve Direction | Right | Left | Right | Left | Left | Right | Right | Left | Left | Right |
| Available width (m) | 3,75 | 1,75 | 3,75 | 1,75 | 1,75 | 3,75 | 1,75 | 3,75 | 1,75 | 3,75 |
| Lateral Clearance (M) | 2,33 | 3,14 | 0,48 | 0,68 | 1,67 | 2,22 | 2,33 | 3,01 | 0,59 | 0,91 |
| Curve Widening (m) | none | 1,61 | none | none | none | none | 0,58 | none | none | none |

Source: Final Report, DED of the Panorama 1 (Sitinjau Lauik 1) Flyover Project.

Table 3. Summary Results of the Sitinjau Lauik Flyover Planned Bend Widening

(STA 2+164.544 s.d 2+812.722)

| GEOMETRIC PARAMETERS | NORMAL | OPPOSITE | NORMAL | OPPOSITE | NORMAL | OPPOSITE | NORMAL | OPPOSITE | NORMAL | OPPOSITE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | STA 2+164.544 |  | STA 2+351.713 |  | STA 2+630.723 |  | STA 2+748.709 |  | STA 2+812.722 |  |
| Design Speed (Vr) | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 40 |
| topping Sight Distance (JPH/SS) | 43 | 54 | 43 | 54 | 43 | 54 | 43 | 54 | 43 | 54 |
| Curve Radius (R) | 250 | 250 | 235 | 235 | 250 | 250 | 2800 | 2800 | 400 | 400 |
| Inner Radius (Ri) | 251,75 | 244,75 | 236,75 | 229,75 | 244,75 | 251,75 | 2794,75 | 2801,75 | 401,81 | 398,19 |
| Curve Length (Lc) | 99,84 | 99,84 | 168,94 | 168,94 | 117,14 | 117,14 | 30,41 | 30,41 | 66,68 | 66,68 |
| Lane width | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 | 3,5 |
| Outer Shoulder Width | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Curve Direction | Right | Left | Right | Left | Left | Right | Left | Right | Right | Left |
| Available width (m) | 3,75 | 1,75 | 3,75 | 1,75 | 1,75 | 3,75 | 1,75 | 3,75 | 1,75 | 3,75 |
| Lateral Clearance (M) | 0,92 | 1,49 | 0,98 | 1,58 | 0,94 | 1,45 | 0,08 | 0,13 | 0,58 | 0,92 |
| Curve Widening (m) | none | none | none | none | none | none | none | none | none | none |

Source: Final Report, DED of the Panorama 1 (Sitinjau Lauik 1) Flyover Project.

Table 4. Summary of Vertical Alignment Design Results for the Proposed Sitinjau Lauik Flyover

| TITIK | STATION | ELEVASI | STATIONING |  | ELEVASI |  | V | g1% | g2% | A | LV | EV | JENIS | K |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PPV | PPV | (m) | PLV | PTV | PLV | PTV | (km/Jam) |  |  | (m) | () | (m) | LENGKUNG |  |
| 1 | 0+000.000 | 379.211 | -0+025.000 | 0+025.000 | 379.204 | 379.219 | 40 | 3.00% | 3.00% | 0.0000 |  | 0.000 | CEKUNG |  |
| 2 | 0+219.769 | 385.804 | 0+201.769 | 0+237.769 | 385.799 | 385.818 | 40 | 3.00% | 8.00% | 0.050 | 50 | 0.003 | CEKUNG | 10.00 |
| 3 | 0+425.000 | 402.222 | 0+375.000 | 0+475.000 | 402.182 | 402.217 | 40 | 8.00% | -1.00% | 0.0900 | 36 | 0.004 | CEMBUNG | 4.000 |
| 3 | 0+504.974 | 401.422 | 0+474.974 | 0+534.974 | 402.425 | 402.436 | 40 | -1.00% | 4.50% | 0.0550 | 60 | 0.003 | CEKUNG | 10.909 |
| 4 | 0+911.492 | 419.716 | 0+881.492 | 0+941.492 | 419.703 | 419.731 | 40 | 4.50% | 5.00% | 0.0050 | 100 | 0.001 | CEKUNG | 200.000 |
| 5 | 1+239.742 | 436.128 | 1+189.742 | 1+289.742 | 436.103 | 436.148 | 40 | 5.00% | 4.00% | 0.0100 | 60 | 0.001 | CEMBUNG | 60.000 |
| 6 | 1+393.741 | 442.288 | 1+343.741 | 1+443.741 | 442.268 | 442.328 | 40 | 4.00% | 8.00% | 0.0400 | 100 | 0.005 | CEKUNG | 25.000 |
| 7 | 1+687.625 | 465.799 | 1+637.625 | 1+737.625 | 465.759 | 465.824 | 40 | 8.00% | 5.00% | 0.0300 | 100 | 0.004 | CEMBUNG | 33.333 |
| 8 | 2+144.314 | 488.633 | 2+114.314 | 2+174.314 | 488.618 | 488.657 | 40 | 5.00% | 8.00% | 0.0300 | 100 | 0.004 | CEKUNG | 33.333 |
| 9 | 2+453.387 | 513.359 | 2+415.137 | 2+491.637 | 513.328 | 513.376 | 40 | 8.00% | 4.50% | 0.0350 | 60 | 0.003 | CEMBUNG | 17.143 |
| 10 | 2+604.426 | 520.156 | 2+584.426 | 2+624.426 | 520.147 | 520.182 | 40 | 4.50% | 13.00% | 0.0850 | 76.5 | 0.008 | CEKUNG | 9.000 |
| 11 | 2+708.079 | 533.631 | 2+708.079 | 2+708.079 | 533.631 | 533.631 | 40 | 13.00% | 12.00% | 0.0100 | 40 | 0.001 | CEMBUNG | 40.000 |
| 12 | 2+774.152 | 541.560 | 2+754.152 | 2+794.152 | 541.536 | 541.584 | 40 | 12.00% | 12.00% | 0.0000 |  | 0.000 | CEKUNG |  |
| 13 | 2+829.695 | 548.225 | 2+829.695 | 2+829.695 | 548.560 | 548.560 | 40 | 12.00% | 7.13% | 0.0487 | 40 | 0.002 | CEMBUNG | 8.215 |
| 14 | 2+899.299 | 553.188 | 2+899.299 | 2+899.299 | 553.188 | 553.188 | 40 | 7.13% |  |  |  |  |  |  |

Source: Final Report, DED of the Panorama 1 (Sitinjau Lauik 1) Flyover Project.

3.4. 	Ex-Ante Evaluation Synthesis

Based on the overall evaluation of horizontal alignment, vertical alignment, and stopping sight distance, it is quantitatively determined that 100% of the primary geometric parameters comply with the minimum requirements of PDGJ 2021 for a design speed of 40 km/h. No curve radius is smaller than the prescribed minimum Rmin=57m. All superelevation values range from 2.10% to 5.20%, remaining below the maximum allowable limit of 8%, while SSD values range from 43 m to 54 m, satisfying the minimum requirement. Similarly, for vertical alignment, the vertical curve lengths (Lv), K-values, and both sag and crest configurations remain within the permissible design parameters.

However, approximately 20% of the curved segments (with radii of 97 m and 110 m) exhibit the smallest geometric margins, with R/R_min ratios of 1.70 and 1.93, respectively, placing them relatively close to the minimum threshold compared to other curves. In addition, several segments exhibit grade variations of up to ±13% and SSD values at the lower bound (43 m), which may reduce operational safety margins, particularly during extended downgrades and at locations with horizontal curvature.

The ex-ante Approach adopted in this study enables early identification of segments with higher potential risk, particularly where small radii, steep grades, and near-minimum SSD values occur simultaneously. The findings indicate that although the design complies with technical standards on a normative basis, variations in safety margins across segments warrant attention, especially in relation to heavy vehicle operations and the extreme topographic conditions of Sitinjau Lauik.

These findings underscore that geometric safety evaluation at the planning stage should not be limited to verifying minimum standard compliance but should also assess the adequacy of safety margins to anticipate risks arising from the interaction of geometric elements along alignments traversing complex terrain.

- Conclusion

The ex-ante geometric safety evaluation of the proposed Sitinjau Lauik Flyover demonstrates that the planned alignment generally complies with the minimum requirements of PDGJ 2021 for a design speed of 40 km/h. The horizontal alignment analysis indicates that all curve radii, ranging from 97 m to 2800 m, exceed the minimum required radius, while the applied superelevation values (2.10%–5.20%) and stopping sight distances (43–54 m) satisfy the prescribed standards. Similarly, the vertical alignment evaluation shows that longitudinal grades, vertical curve lengths, and K-values remain within the acceptable design criteria. Nevertheless, the assessment reveals that several locations possess relatively limited safety margins, particularly at segments characterized by small curve radii (97 m and 110 m), steep grades approaching 13%, and stopping sight distances close to the minimum threshold. The interaction of these geometric elements may influence operational safety, especially for heavy vehicles travelling on prolonged downgrade sections. Therefore, although the flyover design fulfills the normative requirements of PDGJ 2021, the findings highlight the importance of considering safety margin adequacy in addition to standard compliance. Furthermore, the ex-ante Approach applied in this study has proven effective in identifying potential safety risks during the planning stage, thereby supporting the implementation of preventive safety measures before construction and operation.

Future studies are recommended to incorporate vehicle dynamic simulation, operating speed analysis, and heavy-vehicle performance assessment to further validate the operational safety of the proposed flyover under actual traffic conditions.

Acknowledgements

The authors would like to express their sincere appreciation to the institutions that provided access to the geometric design data of the Sitinjau Lauik Flyover. Gratitude is also extended to academic advisors and colleagues for their valuable input, guidance, and scholarly support throughout the preparation of this study.

References

[1]	World Health Organization, “Global Status Report on Road Safety 2018,” Geneva, 2018.

[2]	American Association of State Highway and Transportation Officials, A Policy on Geometric Design of Highways and Streets, 7th ed. Washington, DC, 2018.

[3]	World Road Association, “Road Safety Manual,” Pairs, 2019.

[4]	C. A. O’Flaherty and A. Hughes, Highway Engineering, 5th ed. Oxford, 2016.

[5]	R. A. Krammes et al., “Horizontal alignment design consistency for rural two-lane highways,” Transp. Res. Rec., no. 1500, pp. 7–16, 1995.

[6]	K. Fitzpatrick, L. Elefteriadou, D. W. Harwood, J. M. Collins, J. C. McFadden, and I. B. Anderson, “Speed prediction for two-lane rural highways,” Washington, DC, 2000.

[7]	S. Cafiso, A. Di Graziano, and G. Pappalardo, “Using operating speed consistency to evaluate safety of two-lane rural highways,” Accid. Anal. Prev., pp. 117–124, 2017.

[8]	Y. Chen, S.; Zhou, J.; Wang, “Geometric design consistency and crash prediction modeling,” J. Transp. Eng., vol. 145, no. 6, 2019.

[9]	A. Calvi, “A study on driving performance along horizontal curves of two-lane rural roads,” J. Transp. Saf. Secur., vol. 11, no. 4, pp. 421–438, 2019.

[10]	F. Lord, D.; Mannering, “The statistical analysis of crash-frequency data: A review and assessment,” Transp. Res. Part A, vol. 44, no. 5, pp. 291–305, 2010.

[11]	Austroads, “Guide to Road Safety Part 6: Road Safety Audit,” Sydney, 2016.

[12]	European Commission, “Road Safety Audit and Road Safety Inspection Guidelines,” Brussels, 2013.

[13]	T. Elvik, A. Høye, M. Vaa, and S. Sørensen, The Handbook of Road Safety Measures, 2nd ed. Bingley: Emerald Group Publishing, 2009.

[14]	M. et al. Montella, “Road safety audits: Effectiveness and best practices,” Saf. Sci., vol. 100, pp. 135–145, 2017.

[15]	Direktorat Jenderal Bina Marga, “Pedoman Desain Geometrik Jalan (PDGJ) 2021,” Jakarta, Indonesia, 2021.

[16]	Direktorat Jenderal Bina Marga, “Pedoman Audit Keselamatan Jalan,” Jakarta, Indonesia, 2014.

[17]	R. Lamm, B. Psarianos, and T. Mailaender, Highway Design and Traffic Safety Engineering Handbook. New York, NY: McGraw-Hill, 1999.

[18]	P. Weijermars, Wendy; Wesemann, “Road safety forecasting and ex-ante evaluation of policy in the Netherlands,” Transp. Res. Part A Policy Pract., vol. 52, pp. 64–72, 2013, doi: 10.1016/j.tra.2013.06.001.

[19]	W. Wegman, F.; Berg, H.-Y.; Cameron, I.; Thompson, C.; Siegrist, S.; Weijermars, “Evidence-based and data-driven road safety management,” IATSS Res., vol. 39, 2015, doi: 10.1016/j.iatssr.2015.04.001.

[20]	Kementerian Pekerjaan Umum. Direktorat Jenderal Bina Marga, “Tata Cara Perencanaan Geometrik Jalan Antar Kota,” Jakarta, Indonesia, 1997.

[21]	M. Jiang, H.; Zhou, W.; Liu, C.; Zhang, G.; Hu, “Safe and Ecological Speed Control for Heavy-Duty Vehicles on Long–Steep Downhill and Sharp-Curved Roads,” Sustainability, vol. 12, no. 17, 2020, doi: 10.3390/su12176813.

[22]	K. Moomen, M.; Rezapour, M.; Raja, M. N.; Ksaibati, “Evaluating the Safety Effectiveness of Downgrade Warning Signs on Vehicle Crashes on Wyoming Mountain Passes,” Cogent Eng., vol. 6, 2019, doi: 10.1080/23311916.2019.1580405.

[23]	A. et al. Pérez-Zuriaga, “Preventive safety assessment in road design stages,” Saf. Sci., 2020.

[24]	K. Wang, Y.; Xu, “Impact of vertical alignment on highway safety performance,” Accid. Anal. Prev., 2020.

[25]	F. Russo, E.; Busiello, G.; Dell’Acqua, “Safety performance evaluation of horizontal curves on rural roads,” Transp. Res. Rec., 2021.

[26]	F. Bella, “Driver perception-based design consistency evaluation on two-lane rural roads,” Accid. Anal. Prev., vol. 110, pp. 49–58, 2018.